Task 2

UART

Important Terms

1.Baud Rate: Communication speed (bits/sec)

2.TX: Transmit pin

3.RX: Receive pin

4.Start Bit: Signals data frame start

5.Stop Bit: Signals data frame end

6. Data Frame: Start + Data (5-9 bits) + Parity + Stop

I2C

Important Terms

1.SDA: Serial Data (bidirectional)

2.SCL: Serial Clock (from master)

3.Master: Initiates communication

4.Slave: Responds to master

5.Address: 7-bit device identifier

6.ACK/NACK: Acknowledge signal

SPI

Important Terms

1.SDA: Serial Data (bidirectional)

- SCL: Serial Clock (from master)

3.Master: Initiates communication

4.Slave: Responds to master

5. Address: 7-bit device identifier

6.ACK/NACK: Acknowledge signal

Theoretical Difference between UART,I2C,SPI

UART

- It is asynchronous serial communication

- We do not require any clock.

- Uses 2 wires transmit and receive.

- It is used for point to point data transfer.

- Used in Serial monitor, Bluetooth, GPS modules.

I2C

- It is synchronous serial protocol.

- It requires Serial Clock Line(SCL).

- Wires used are Serial Data Line (SDA) and Serial Clock Line(SCL).

- It allows multiple devices to communicate.

- Used in Sensors , Real time clock(RTC),Electrical Erasable Programmable Read Only Memory (EEPROM) .

SPI

- It is high speed synchronous serial protocol.

- It requires serial clock line (SCL).

- Wires used are Master out slave in (MOSI), Master in slave out(MISO) , Serial clock (SCL) , SS / CS – Slave Select / Chip Select.

- Used for short distance communication.

- Used in SD cards, Displays ,ADCs.

Practical Difference between UART,I2C,SPI

UART

- Data transfer depends on baud rate matching.

- Communication fails if TX/RX are not crossed.

- No clock signal observed.

- Simple and easy to debug using Serial Monitor.

I2C

1.  Only two wires used for multiple devices.

2. Devices respond based on address.

3.  ACK/NACK bits observed during communication.

4.  Pull-up resistors are necessary for SDA and SCL.

SPI

1.  Fastest data transfer among the three.

2. Chip Select (CS) controls active slave.

3. Separate data lines allow full-duplex transfer.

4. More wiring compared to UART and I²C.

Simulation Screenshots

UART

I2C

SPI

Working Codes

UART

// Pin definitions

const int POT_PIN = A0;  // Potentiometer connected to analog pin A0

// Variables for sensor reading

int potValue = 0;

float voltage = 0.0;

int percentage = 0;

void setup() {

// Initialize UART communication at 9600 baud rate

Serial.begin(9600);

while (!Serial) {

;

}

Serial.println("====================================");

Serial.println("  UART Serial Communication Demo");

Serial.println("  Potentiometer Analog Reading");

Serial.println("====================================");

Serial.println();

Serial.println("Reading potentiometer values...");

Serial.println();

analogReference(DEFAULT);

delay(1000);

}

void loop() {

potValue = analogRead(POT_PIN);

voltage = potValue * (5.0 / 1023.0);

percentage = map(potValue, 0, 1023, 0, 100);

Serial.println("--- New Reading ---");

Serial.print("Raw ADC Value: ");

Serial.println(potValue);

Serial.print("Voltage: ");

Serial.print(voltage, 2);

Serial.println(" V");

Serial.print("Percentage: ");

Serial.print(percentage);

Serial.println(" %");

Serial.println();

delay(1000);

Serial.print(millis());      // Timestamp

Serial.print(",");

Serial.print(potValue);      // Raw value

Serial.print(",");

Serial.print(voltage, 2);    // Voltage

Serial.print(",");

Serial.println(percentage);  // Percentage

I2C

Arduino(Master)

#include <Wire.h>

#define SLAVE_ADDRESS 0x08

int receivedValue = 0;

float voltage = 0.0;

int percentage = 0;

void setup() {

Wire.begin();

Serial.begin(9600);

Serial.println("======================================");

Serial.println("  I²C Master-Slave Communication");

Serial.println("  Master Arduino - Requesting Data");

Serial.println("======================================");

Serial.println();

Serial.println("Master initialized. Requesting data from Slave...");

Serial.println();

delay(1000);

}

void loop() {

// Request 2 bytes of data from slave at address 0x08

Wire.requestFrom(SLAVE_ADDRESS, 2);

delay(10);

if (Wire.available() >= 2) {

byte highByte = Wire.read();

byte lowByte = Wire.read();

receivedValue = (highByte << 8) | lowByte;

voltage = receivedValue * (5.0 / 1023.0);

percentage = map(receivedValue, 0, 1023, 0, 100);

Serial.println("--- I²C Data Received ---");

Serial.print("Raw ADC Value: ");

Serial.println(receivedValue);

Serial.print("Voltage: ");

Serial.print(voltage, 2);

Serial.println(" V");

Serial.print("Percentage: ");

Serial.print(percentage);

Serial.println(" %");

Serial.println();

} else {

Serial.println("ERROR: No data received from Slave!");

Serial.println("Check I²C connections (SDA, SCL, GND)");

Serial.println();

}

// Request data every 1 second

delay(1000);

}

Arduino(Slave)

#include <Wire.h>

#define SLAVE_ADDRESS 0x08

const int POT_PIN = A0;

int potValue = 0;

void setup() {

Wire.begin(SLAVE_ADDRESS);

Wire.onRequest(requestEvent);

Serial.begin(9600);

Serial.println("======================================");

Serial.println("  I²C Slave Arduino Ready");

Serial.println("  Address: 0x08");

Serial.println("======================================");

Serial.println();

Serial.println("Waiting for Master requests...");

Serial.println("Reading potentiometer on A0");

Serial.println();

}

void loop() {

potValue = analogRead(POT_PIN);

Serial.print("Current Pot Value: ");

Serial.println(potValue);

delay(500);

delay(50);

}

void requestEvent() {

potValue = analogRead(POT_PIN);

byte highByte = (potValue >> 8);

byte lowByte = (potValue & 0xFF);

Wire.write(highByte);

Wire.write(lowByte);

Serial.print("Master requested data. Sent: ");

Serial.print(potValue);

Serial.print(" (Voltage: ");

Serial.print(potValue * (5.0 / 1023.0), 2);

Serial.println(" V)");

}

SPI

Arduino(Master)

#include <SPI.h>

const int SS_PIN = 10;

byte receivedData = 0;

int buttonState = 0;

int buttonPressCount = 0;

unsigned long lastPressTime = 0;

void setup() {

Serial.begin(9600);

pinMode(SS_PIN, OUTPUT);

digitalWrite(SS_PIN, HIGH);  // Deselect slave initially

SPI.begin();

SPI.setClockDivider(SPI_CLOCK_DIV16);  // 1 MHz SPI clock

Serial.println("==========================================");

Serial.println("  SPI Master-Slave Communication");

Serial.println("  Master Arduino - Receiving Button Data");

Serial.println("==========================================");

Serial.println();

Serial.println("Master SPI initialized");

Serial.println("Waiting for button presses on Slave...");

Serial.println();

delay(1000);

}

void loop() {

digitalWrite(SS_PIN, LOW);

delayMicroseconds(10);

receivedData = SPI.transfer(0x00);

delayMicroseconds(10);

digitalWrite(SS_PIN, HIGH);

buttonState = receivedData;

Serial.print("SPI Data Received: 0x");

if (receivedData < 16) Serial.print("0");

Serial.print(receivedData, HEX);

Serial.print(" | Button State: ");

if (buttonState == 1) {

Serial.println("PRESSED ✓");

if (millis() - lastPressTime > 300) {

buttonPressCount++;

lastPressTime = millis();

Serial.print(">>> Button Press Count: ");

Serial.println(buttonPressCount);

}

Serial.println();

} else if (buttonState == 0) {

// Button is released

Serial.println("RELEASED");

} else {

Serial.println("UNKNOWN");

}

delay(200);

}

Arduino(Slave)

#include <SPI.h>

const int BUTTON_PIN = 2;

volatile byte buttonState = 0;

volatile boolean dataRequested = false;

int lastButtonState = 0;

void setup() {

pinMode(BUTTON_PIN, INPUT_PULLUP);

pinMode(MISO, OUTPUT);

Serial.begin(9600);

Serial.println("==========================================");

Serial.println("  SPI Slave Arduino Ready");

Serial.println("  Reading Button on Pin 2");

Serial.println("==========================================");

Serial.println();

Serial.println("Waiting for SPI communication...");

Serial.println("Press the button to send data!");

Serial.println();

SPCR |= _BV(SPE);  // Enable SPI

SPI.attachInterrupt();

delay(1000);

}

ISR (SPI_STC_vect) {

byte received = SPDR;

SPDR = buttonState;

dataRequested = true;

}

void loop() {

int currentButtonState = digitalRead(BUTTON_PIN);

if (currentButtonState == LOW) {

buttonState = 1;  // Button pressed

} else {

buttonState = 0;  // Button released

}

if (currentButtonState != lastButtonState) {

lastButtonState = currentButtonState;

Serial.print("Button State Changed: ");

if (buttonState == 1) {

Serial.println("PRESSED");

} else {

Serial.println("RELEASED");

}

}

if (dataRequested) {

dataRequested = false;

Serial.print("Data sent to Master via SPI: ");

Serial.println(buttonState);

}

delay(50);

}